IBVAP

AI-Based Intelligent Video Analytics Platform for Border Surveillance

Executive Summary  ·  Problem Statement ID 26187  ·  SSB / MHA  ·  Reference architecture + working prototype  ·  September 2026

# 1. What and Why

IBVAP turns SSB's existing border video surveillance — RTSP cameras already installed at Border Outposts (BOPs) and connected over the existing SSB VPN — into an intelligent, 24×7 surveillance system. Deep-learning analytics (YOLOv8, CPU-only) detect person/vehicle intrusions across virtual fences and loitering in restricted areas, and raise an alert on the watch-room console in under 3 seconds, with automatic evidence (snapshot + 20-second clip) for every event. The system needs zero new hardware at the BOP: it is software that runs on a central SSB server (16–32 cores, 64 GB+, 5–10 TB, 1 Gbps) and reuses the existing camera/VPN estate.

A complete, working reference prototype is delivered with this summary: live multi-camera wall, real AI inference on video, virtual-fence editor, real-time WebSocket alerts, searchable event database with evidence, role-based access (admin/operator/viewer), reports & analytics, and an offline BOP deployment map — all deployable via Docker.

# 2. Headline Numbers — proposal target vs prototype evidence

| Metric (proposal §7) | Target | Prototype evidence |
| --- | --- | --- |
| Alert latency (fence crossing → alert) | < 3 s | p50 2 ms · p95 3 ms · max 4 ms — measured end-to-end via real WebSocket (KPI report) |
| Detection accuracy | > 85% | Event precision/recall 100% on ground-truth scene; YOLOv8n person-conf p50 0.81, 31% of detections ≥ 0.85; annotated-set eval in Phase 1 |
| Frames per camera (CPU, 640×360) | 3–5 fps | 16 fps single-stream measured on 2-core VM (YOLOv8n); scales 10 → 1000+ cams via workers/GPU option |
| New BOP hardware | None | None — RTSP + VPN reuse only |
| Event storage | Event-only ≈ 100 GB/mo per 100 cams | Snapshots + 10 s pre-roll clips only; no continuous recording |
| Evidence per event | Snapshot + 10–20 s clip | JPG snapshot + 20 s MP4 (10 s before / 10 s after) |
| Deployment | Docker, PostgreSQL production | Dockerfile + docker-compose; SQLite in prototype, Postgres swap is config-level |
| Access control | RBAC + audit | 3 roles enforced (401/403 verified) + audit log table |

# 3. Architecture in One Breath

Capture: existing RTSP cameras (H.264) over the existing SSB VPN; 640×360 analytics resolution; motion-triggered frame sampling to cut CPU.

Analytics: FastAPI service with per-camera worker threads → YOLOv8n (INT8-capable, CPU) person/vehicle detection → IoU tracker → rule engine (intrusion line / loitering area, per-zone cooldown, dropout tolerance, and track handoff — a tracker ID reset can't fake a re-intrusion).

Alerting: in-process alert hub → WebSocket push to consoles (< 1 s fan-out) with polling fallback; severity = high (intrusion) / medium (loitering).

Evidence: ring buffer of 10 s pre-roll per camera → JPG snapshot + 20 s MP4 clip auto-attached to every event.

Data: events, alerts, cameras, zones, BOPs, users, audit — SQLite in the prototype, PostgreSQL in production (identical schema, config-level swap).

Console: single-page web console (no build step) — live wall with drawn boxes, alert feed, event timeline with search, fence editor, camera registration, reports & analytics, BOP map, admin users.

Scale-out: stateless analytics workers behind a queue; horizontal scale from 10 to 1000+ cameras; GPU option later (OpenVINO/TensorRT); multi-tenant BOP scoping and C2 API are designed-in.

# 4. What Is Delivered

| Deliverable | Status |
| --- | --- |
| Working prototype (4 demo cameras incl. real-AI video, RBAC, zones, events, evidence, alerts) | Built & verified live |
| System architecture document (17 sections) + 5 diagrams (logical, deployment, pipeline, sequence, ER) | Delivered |
| User manual, admin guide, API reference, training script (2-hr bilingual BOP session) | Delivered |
| Automated test suite — 27 tests (geometry, tracker, event engine, full API incl. WebSocket, latency) | All passing (18.7 s) |
| KPI verification report — measured alert latency, event precision/recall, YOLO inference audit | Delivered (docs/KPI_VERIFICATION.md) |
| Docker + docker-compose deployment, .env config, demo asset fetcher | Delivered |
| Reports & analytics endpoint + console (daily trend, type/severity mix, per-camera load) | Built & verified live |
| BOP deployment map (offline schematic, GPS-plotted, status-coloured cameras) | Built in console |
| Budget workbook (6 sheets, ₹) + 14-slide proposal deck (PPTX) | Delivered |

# 5. Phased Roadmap

| Phase | Scope | Exit criteria |
| --- | --- | --- |
| Phase 0 — Pilot (2–3 BOPs, 10–20 cams) | Deploy on existing SSB server; operator training (script provided); weekly tuning of zones | Alert latency < 3 s in field; operator acceptance |
| Phase 1 — Hardening (1–3 months) | PostgreSQL, Nginx TLS, MFA, HA pair, ONNX-INT8 inference, load test to 100 cams | 4-week soak, < 2% false positives per shift after tuning |
| Phase 2 — Scale (3–9 months) | Queue-based workers to 1000+ cams, multi-tenant BOP scoping, GIS map with real basemap, C2 API | Design capacity demonstrated at 500 cams |
| Phase 3 — Advanced analytics (optional) | ANPR, face watchlist, abandoned-object, multi-frame behaviour rules | Feature-by-feature, funded separately |

# 6. Budget at a Glance (₹, indicative)

Full workbook: docs/budget/IBVAP-Budget.xlsx (6 sheets: pilot, scaling, hardware, software/licensing, manpower, summary).

| Head | Pilot (2–3 BOPs) | Full build-out (1000+ cams) |
| --- | --- | --- |
| Software development & integration | ₹ 12–18 L | ₹ 60–90 L (incl. scale-out engineering) |
| Central server (existing SSB estate — uplift only) | ₹ 4–8 L | ₹ 25–40 L (2 nodes HA + storage) |
| Deployment, training, pilot ops (3 months) | ₹ 3–6 L | ₹ 12–20 L |
| Indicative total | ₹ 19–32 L | ₹ 97–150 L |

Consistent with the proposal's pilot band (₹ 2–4 L per BOP for 10–20 cameras) when central costs are amortised across the BOP estate.

# 7. Key Risks and Mitigations

| Risk | Mitigation (already in design) |
| --- | --- |
| VPN bandwidth saturation from 1000+ cam streams | 640×360 analytics resolution + motion-triggered sampling; per-BOP ingest caps; edge pre-filter option in Phase 2 |
| False alarms eroding operator trust | Per-zone cooldown, dropout-tolerant tracker, operator acknowledgement loop; tuning SOP in training script |
| CPU inference not keeping up at scale | INT8/ONNX, worker pool scale-out, GPU option; capacity test at Phase 1 exit |
| Evidence retention & legal chain | Event-only storage, tamper-evident DB records, audit log of every user action |
| Vendor lock-in | 100% open-source stack (PyTorch/Ultralytics, FastAPI, PostgreSQL, FFmpeg, Docker) |

# 8. Immediate Next Steps

Stakeholder review of this summary + live prototype walkthrough (30 min, demo logins provided).

Confirm pilot BOP shortlist (2–3) and central server allocation (16–32 cores / 64 GB / 5 TB minimum).

Sign off Phase 0 scope; 6-week pilot mobilisation using the delivered deployment package.

Prepared for SSB / MHA  ·  IBVAP working prototype + reference architecture  ·  See ARCHITECTURE.md for the full 17-section design  ·  September 2026